ASEAN Customs Transit System Central Service (ACTS CS) Extension Plan

# Terms of References

## INTRODUCTION

ACTS Central Services (ACTS CS) have been operated by ASEAN Secretariat’s Central Management Team (CMT) whose main objective is to ensure uninterrupted operations ACTS CS and support long term sustainability of the ACTS as a whole. Their responsibilities include:

- Managing the roll−out of ACTS to the north−south and east−west corridors;

- Hosting and Managing Central Services: Reference Data System and Management Information System;

- Hosting and Managing Information Portal;

- Hosting and Managing Central Helpdesk;

- Managing ACTS solutions of the ASEAN Member States (AMS).

To fully cover the needs of the ACTS operations and to support CMT’s main objective and their

responsibilities, the following additional environments are required for:

- Training ACTS CS and AMS environments to enable CMT to support ACTS live operation for the six AMS namely (Cambodia, Lao PDR, Malaysia, Singapore, Thailand, and Viet Nam) and remaining AMS interested in joining ACTS in the future and to increase and maintain CMT’s capacity and knowledge by having dedicated training environments for AMS, CMT’s existing staff and future staff of CMT; and

- Testing, Reference and Staging ACTS CS and AMS environments to support deployment testing, User Acceptance Testing (UAT) and Conformance Testing (CT) of ACTS’s new software releases before they are approved for live operations and to support CMT’s Central Helpdesk duties.

As requested by the Trade and Facilitation Division of ASEC which responsible for the effectiveness and operational of ACTS, ARISE Plus Grant wishes to seek a qualified services provider for hosting the aforementioned additional environments for a total of 12 consecutive months

## HOSTING SERVICE REQUIREMENTS

  - Technical Experiences and qualification requirement

    - Service provider must have at least 3 years’ experience for hosting the virtual servers in the private cloud or in the cloud for the following activities:

      - Provisioning and installing the virtual servers

      - Configuration of the virtual servers

      - Data backup for the virtual servers

    - Service provider must have qualified engineers to deliver the services. Engineers must be certified for the following systems:

      - Linux OS

      - Windows OS

      - Networking e.g. CCNA

  - Technical Scope of Work

2.2.1	Scope of Works for ACTS CS Extended Services

| No. | Requirement details | Status | Notes |
| --- | --- | --- | --- |
|  | General |  |  |
| 1 | Provide IT infrastructure components with guaranteed computing power and capacity in virtual environments OR cloud. The following infrastructure components must be provided: - computing/virtual server components - network and connection components - security components - storage components | Mandatory | Detail requirements appear in Chapter 2.3 |
| 2 | In case of virtual server setup hosted in a private data center, IT infrastructure components must hosted in the TIA 942 certified data center at least Tier III specification. The Service provider must also inform the compliance of the data center international certification standard if the virtual server hosted in the public data center (cloud). | Mandatory | Certificate of Standard Conformity must be provided (Specification appears in ANNEX 1) |
| 3 | Dedicated internet international connection with minimum bandwidth 1 Mbps Downstream and 1 Mbps Upstream. Service provider must provide IP Public for each virtual server based on the application requirement. | Mandatory | Service provider must provide the burst bandwidth policy |
| 4 | IT governance and security compliance (e.g.: ISO 27000 series) | Mandatory | Certificate of Standard Conformity must be provided (Specification appears in ANNEX 2) |
|  | Operating System Managed Service |  |  |
| 5 | Provide Operating System (OS) as required by software platforms and ACTS applications | Mandatory | Detail requirements appear in Chapter 2.3 |
| No. | Requirement details | Status | Notes |
| 6 | Provide the Migration services as required by ACTS Applications. | Mandatory |  |
|  | Network and Security Managed Service |  |  |
| 7 | Provide network and security as required by systems. The following network and security components must be provided: - Network zoning - VLAN creation and management - Firewall connection management | Mandatory | Detail requirements appear in Chapter 2.3 |
|  | Monitoring and Administration |  |  |
| 8 | Accessibility of the IT infrastructure components through internet connection. | Mandatory |  |
|  | Service Level Agreement (SLA) |  |  |
| 9 | Provide the service based on required SLA for availability, incident and problem resolution | Mandatory | Detail requirements appear in Chapter 2.4 |

  - Service Architecture and Quantity

ACTS CS Extension will comprise of 2 environments e.g. Testing and Training/Staging. The following figures are the architecture for each of the environment.

Figure 1. ACTS CS Extension Environment

Detail server specification of each environment of the ACTS CS extension service will be explained on the below table.

Table 1. Server Specification of Testing Environment on ACTS CS Extension

| Server Name (Testing environment) | Type | Operating System | CPU Cores | RAM (Gb) | Hard Disk Capacity (Gb) |
| --- | --- | --- | --- | --- | --- |
| TestVM#1 | Virtual | CentOS 7 64bit | 4 | 8 | 1024 |
| TestVM#2 | Virtual | CentOS 7 64bit | 8 | 16 | 1024 |
| TestVM#3 | Virtual | CentOS 7 64bit | 4 | 8 | 1024 |

Table 2. Server Specification of Training and Staging Environment on ACTS CS Extension

| Server Name (Staging environment) | Type | Operating System | CPU Cores | RAM (Gb) | Hard Disk Capacity (Gb) |
| --- | --- | --- | --- | --- | --- |
| StageVM#4 | Virtual | CentOS 7 64bit | 4 | 8 | 1024 |
| StageVM#5 | Virtual | CentOS 7 64bit | 8 | 16 | 1024 |
| StageVM#6 | Virtual | CentOS 7 64bit | 4 | 8 | 1024 |

Specifically, for the Central Services Extension Environment the following hardware performance requirements have to be met:

- MINIMUM recommended IOPS for the servers storage should be, on local disks ~800 IOPS, on SAN/NAS disks ~1800 IOPS.

- MAXIMUM recommended Latency for the servers storage should be, on local disks ~3 ms, on SAN/NAS disks ~2 ms.

- MAXIMUM recommended Seek Time for the servers storage should be, on local disks ~0.2 ms, on SAN/NAS disks ~0.1 ms.

  - Service Level Agreement (SLA)

The ACTS CS hosting service must deliver under following Service Level Agreement:

| SLA Criteria | Description | SLA Target |
| --- | --- | --- |
| Service availability | Availability of the CS hosting service | Minimum 99 % |
| Operational | Operational time of the helpdesk | ASEAN normal working time operations, with 24/7 critical issues hotline |
| Call response time | Time to acknowledge a Call | Maximum 2 working hours (1 hour for critical call) |

| SLA Criteria | Description | SLA Target |
| --- | --- | --- |
| “Critical” problem resolution time | Time to resolve (repair) the problem and continue operation | Maximum 8 hours |
| “Critical” problem resolution update time | Time to give regular update on the “critical” problem | Every 1 hour until resolve |
| “Normal” problem resolution time | Time to resolve (repair) the problem | Maximum 4 working days |
| “Normal” problem resolution update time | Time to give regular update on the “normal” problem | Every 1 working day until resolve |
| “Low” problem resolution time | Time to resolve (repair) the problem | Maximum 14 working days |
| “Low” problem resolution update time | Time to give regular update on the “low” problem | Every 4 working days until resolve |

Definition:

- Critical problem, faults that seriously impair or halt performance of the system, with concerns on the following conditions:

  - System, server or critical application down

  - The system is running, but with repeated interruptions

  - The user cannot make use of a ‘must−have’ business−essential function in the production system

  - The problem cannot be solved by a restart or a bypass or a workaround

- Normal problem, faults that affect productivity or development of the system, with concerns on the following conditions:

  - Problems which do not have a significant impact on current productivity

  - The production system is running, but with limitations

  - A function in the production system is failing, but there is a bypass available

- Low problem, minor faults which do not affect the use of the system and issues with low priority.

## Expected Tasks, Timelines and Deliverables

| Timelines | Expected Tasks | Expected Deliverables |
| --- | --- | --- |
| Ten (10) days upon Vendor awardee | Undertake Migration process | Cloned and functioning set of ACTS CS Extension. |
| Seven (7) days upon completing the migration process. | Testing phase and cut-off date, migration report | Submission of Migration report. |
| Contract Period | Maintain a stable environment for ACTS CS Extension Service | Deliver Managed Service for ACTS CS Extension services according to SLA. |

## Requirement for the Submission

The prospective bidder is required to submit the following:

- Technical

Interested candidates including consulting firms and individual consultants are invited to submit ‘Technical Proposal’ package in English including the following documents and requirements.

  - CV and or Certification(s) to fulfill the requirements on point 2.1 on this TOR.

  - Compliance checklist on the requirements point 2.2 and SLA under 2.4 on this document.

  - Provide the service based on point 2.3 and agreed on timelines on point 3.

- Financial

A detailed financial proposal indicating the costs (IDR for local bidders and USD for international bidders) required for undertaking the assignment. The payment schedule follows:

- 1st payment 50% upon PO & Agreement signing.

- 2nd payment 30% upon completion of the 3rd month of service.

- Final payment 20% upon completion and acceptance of work.

# ANNEX A

## DATA CENTER SPECIFICATION

A data center is a centralized repository, either physical or virtual, for the storage, management, and dissemination of data and information organized around a particular body of knowledge or pertaining to a particular business. It secures the installed servers and components in use, protects them from external dangers, and provides the infrastructure required for continued reliable operation. The physical structure provides protection against unauthorized access, fire and natural disasters. The necessary power supply and climate control ensure reliability.

Accurate planning as well as sustainable operation of a modern data center are steps companies need to take to meet the network security standards requirements. The enormous infrastructural requirements involved lead many companies to opt for an external supplier and outsource their data center needs.

In April 2005, the TIA responded with the TIA−942 Telecommunications Infrastructure Standards for Data Centers, the first standard to specifically address data center infrastructure. Intended for use by data center designers early in the building development process, TIA−942 encompasses site space and layout, cabling infrastructure, tiered reliability, and environmental considerations.

## Tier-I: Basic Data Center Site Infrastructure

A Tier−I site is a basic data center, with non−redundant capacity components (e.g. generator, UPS, or chiller) and a non−redundant electrical distribution path serving the computer equipment. A Tier−I site is also required to have a minimum of twelve hours of fuel storage for onsite generation in the event of utility loss. A Tier−I data center is a significant step up from a server room or IT closet, which may not have generators, UPS, or other common data center equipment.

## Tier-II: Redundant Data Center Site Infrastructure Capacity Components

A Tier−II site builds upon the requirements of a Tier−I site and adds redundancy to capacity components, such as adding an additional UPS (either an extra UPS in a module−redundant configuration or an extra UPS in a UPS−redundant configuration), or by including redundant CRACs beyond what is required to cool the load. These redundant capacity components will avoid some outages and speed service restoration in other outages compared to a Tier−I site.

A Tier II data center has redundant capacity components (N+1) and single non−redundant distribution path for power and cooling distribution and 30 % raised floors are required.

## Tier-III: Concurrently Maintainable Data Center Site Infrastructure

A Tier−III site builds upon the requirements of a Tier−II site and adds multiple independent electrical and cooling distribution paths serving the computer equipment. All computer equipment must have multiple power sources, either through multiple power supplies or front− ended by a static switch connected to different UPSs. The key requirement for a Tier−III site is that each and every capacity component and element in the distribution paths can be removed for service on a planned basis without impacting any of the computer equipment (servers housing

inside the data center). This is critical because no computer equipment outage is required to perform infrastructure maintenance.

## Tier-IV: Fault Tolerant Data Center Site Infrastructure

A Tier−IV site builds upon Tier−III and in addition requires that multiple distribution paths must be active at all times, the distribution paths must be physically isolated from one another (compartmentalization), and that cooling must always be available − even in a the event of a loss of utility or other failure. The key added value from a Tier−IV site is that a single failure of any capacity system, capacity component, or distribution element will not impact the computer equipment, and that the system is automated to respond to a failure in the system.

TIA−942 is the comprehensive Data Centre standard that has been developed by the Telecom Industry Association, USA, and is the most widely recognized Data Centre standard in the world.

The American standard is currently the most comprehensive body of standards for data centers. Not only is cabling described, but also the entire data center among other things, the datacenter is divided up into so−called “tiers”, or different classes of availability. The data center must satisfy specific requirements for each class in order to guarantee availability.

## TIA-942 Requirements

| Tier requirements |  | Tier−I | Tier−II | Tier−III |  |  | Tier−IV |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Distribution paths power cooling | and | 1 | 1 | 1 active alternate | / | 1 | 2 active |
| Redundancy active Components |  | N | N+1 | N+1 | 2N+1 |  |  |
| Support space to |  | 20 % | 30% | 80−90% | 100% |  |  |

raised	floor ration

| Raised floors | 12 inches | 18 inches | 30−36 inches | 30−36 inches |
| --- | --- | --- | --- | --- |
| Floor Loading | 85 | 100 | 150 | 150+ |
| Initial build out per Cabinet | <1 kW | 1−2 kW | 1−2 kW | 1−3 kW |
| Ultimate kW / Cabinet (typical) | <1 kW | 1−2 kW | >3 kW | >4kW |
| Utility Voltage | 208, 480 | 208, 480 | 12−15kV | 12−15kV |
| Annual site down time | 28.8 hours | 22 hours | 1.6 hours | 0.4 hours |
| Concurrently Maintainable | No | No | Yes | Yes |

SPOF (Single

Many + Human

Many + Human

Some + Human

Fire,	EPO	+

| Points Of Failures) | errors | errors | errors | Human errors |
| --- | --- | --- | --- | --- |
| Staffing shifts Staff / shift | None None | 1 shift | 1+shift 1−2 / shift | 24 by forever 2+ / shift |
| Availability | 99.671% | 99.749% | 99.982% | 99.995% |

## TIA-942 Standards

In terms of space needed to meet TIA−942 standards:

  - Entrance Room (ER): the location for access provider equipment and demarcation points, as well as the interface with campus cabling and carrier entrance facilities

  - Main Distribution Area (MDA): the location of main cross−connect (MC)

  - Horizontal Distribution Area (HDA): the location of horizontal cross−connect (HC)

  - Zone Distribution Area (ZDA): the location of zone outlet (ZO) or consolidation point (CP)

  - Equipment Distribution Area (EDA): the location of equipment cabinets and racks Ideally, separate rooms for each point listed above should be available for use, but this is not realistic and is often consolidated to defined areas. Careful planning is needed to upgrade the existing ASEC server room to TIER−III. For example, since the system is already setup and running, the service cannot be interrupted and the space is limited.

## TIA-942 Average Space Layout

| Data type | center | Server cabinet | Server room | Small data center | Mid-size data center | Large data center |
| --- | --- | --- | --- | --- | --- | --- |
| # of servers |  | 4.8 | 19 | 150 | 600 | 6000 |
| Total power |  | 1.9 kW | 11 kW | 105 kW | 550 kW | 5,700 kW |
| Floor size |  | 5 m2 | 20 m2 | 150 m2 | 600 m2 | 6,000 m2 |

ANNEX B

## ISO/IEC 27002 (Information Security)

The majority of errors are due to human error − avoiding this requires implementation of IT governance in line with international recognized operational procedures such as ISO/IEC 27002.

The ISO/IEC 27002 catalog includes:

    - Framework

    - Acceptable Use of Information Technology Resources

    - Information Security Definition & Terms

    - Risk assessment

    - Security policy − management direction

    - Organization of information security − governance of information security

    - Asset management − inventory and classification of information assets

    - Human resources security − security aspects for employees joining, moving and leaving an organization

    - Physical and environmental security − protection of the computer facilities

    - Communications and operations management − management of technical security controls in systems and networks

    - Access control − restriction of access rights to networks, systems, applications, functions and data

    - Information systems acquisition, development and maintenance − building security into applications

    - Information security incident management − anticipating and responding appropriately to information security breaches

    - Business continuity management − protecting, maintaining and recovering business− critical processes and systems

    - Compliance − ensuring conformance with information security policies, standards, laws and regulations

The information security controls clearly defined within ISO/IEC 27002 are generally regarded as the best practice means of operational procedures. Some typical examples found in the catalog are as follows1:

## Physical and Environmental security

  - Physical access to premises and support infrastructure (communications, power, air conditioning etc.) must be monitored and restricted to prevent, detect and minimize the effects of unauthorized and inappropriate access, tampering, vandalism, criminal damage, theft etc.

  - The list of people authorized to access secure areas must be reviewed and approved periodically (at least once a year) by Administration or Physical Security Department, and crosschecked by their departmental managers.

1Note: these examples are merely an illustration.

  - Photography or video recording is forbidden inside Restricted Areas without prior permission from the designated authority.

  - Suitable video surveillance cameras must be located at all entrances and exits to the premises and other strategic points such as Restricted Areas, recorded and stored for at least one month, and monitored around the clock by trained personnel.

  - Access cards permitting time−limited access to general and/or specific areas may be provided to trainees, vendors, consultants, third parties and other personnel who have been identified, authenticated, and authorized to access those areas.

  - Other than in public areas such as the reception foyer, and private areas such as rest rooms, visitors should be escorted at all times by an employee while on the premises.

  - The date and time of entry and departure of visitors along with the purpose of visits must be recorded in a register maintained and controlled by Site Security or Reception.

  - Everyone on site (employees and visitors) must wear and display their valid, issued pass at all times, and must present their pass for inspection on request by a manager, security guard or concerned employee.

  - Access control systems must themselves be adequately secured against unauthorized/inappropriate access and other compromises.

  - Fire/evacuation drills must be conducted periodically (at least once a year).

## Human Resource security

  - All employees must be screened prior to employment, including identity verification using a passport or similar photo ID and at least two satisfactory professional references. Additional checks are required for employees taking up trusted positions.

  - All employees must formally accept a binding confidentiality or non−disclosure agreement concerning personal and proprietary information provided to or generated by them in the course of employment.

  - Human Resources department must inform Administration, Finance and Operations when an employee is taken on, transferred, resigns, is suspended or released on long− term leave, or their employment is terminated.

  - Upon receiving notification from HR that an employee's status has changed, Administration must update their physical access rights and IT Security Administration must update their logical access rights accordingly.

  - An employee's manager must ensure that all access cards, keys, IT equipment, storage media and other valuable corporate assets are returned by the employee on or before their last day of employment

## Access control

  - Users of corporate IT systems, networks, applications and information must be individually identified and authenticated.

  - User access to corporate IT systems, networks, applications and information must be controlled in accordance with access requirements specified by the relevant Information Asset Owners, normally according to the user's role.

  - Generic or test IDs must not be created or enabled on production systems unless specifically authorized by the relevant Information Asset Owners.

  - After a predefined number of unsuccessful logon attempts, security log entries and (where appropriate) security alerts must be generated and user accounts must be locked out as required by the relevant Information Asset Owners.

  - Passwords or pass phrases must be lengthy and complex, consisting of a mix of letters, numerals and special characters that would be difficult to guess.

  - Passwords or pass phrases must not be written down or stored in readable format.

  - Authentication information such as passwords, security logs, security configurations and so forth must be adequately secured against unauthorized or inappropriate access, modification, corruption or loss.

  - Privileged access rights typically required to administer, configure, manage, secure and monitor IT systems must be reviewed periodically (at least twice a year) by Information Security and cross−checked by the appropriate departmental managers.

  - Users must either log off or password−lock their sessions before leaving them unattended.

  - Password−protected screensavers with an inactivity timeout of no more than 10 minutes must be enabled on all workstations/PCs.

  - Write access to removable media (USB drives, CD/DVD writers etc.) must be disabled on all desktops unless specifically authorized for legitimate business reasons.

[Annex II – Payment Reduction]

Downtime tolerance per month

24 hr * 30 Days * (100%-99%) = 7,2 hr(s)

Billing reduction calculations

((N – 0,72) / 720) * A

N 	= Total downtime (Hours) in a month

720 	= Total hours of 30 days

A	= Monthly services fee before tax (VAT)