TERMS OF REFERENCE

GENERAL CHECK FIRE ALARM SYSTEM AT ASEAN SECRETARIAT NEW BUILDING

- Background

The ASEAN Secretariat was established in February 1976 by the Foreign Ministers of ASEAN. It was first housed at the Department of Foreign Affairs of Indonesia in Jakarta and moved to the current location at 70A Jalan Sisingamangaraja, Jakarta since 1981.   Currently, the ASEAN Secretariat (ASEC) is housed at two buildings:

- The Heritage Building since 1981. The Building has 9 stories with approximately 11,000 sqm in total areas.

- The New Building since 2019.  The Building has 16 stories with approximately 44,000 Sqm in total areas.  It is located adjacent to the Heritage Building.

In order to improve building security in terms of the rapid response of the building's alarm system, it is necessary to thoroughly check the existing alarm system in the new building.

- Objectives

Ensure the building's alarm system is functioning and running normally through proper inspection adjustment and repair.

- Expected Deliverables

- All smoke detectors in each room are functioning properly

- All control modules on each floor of the building are functioning properly

- Training session and manual for reference

- Scope of Works

The general scope of work consists of but not limited to:

- Contractor shall provide fire alarm system maintenance/repair/testing services

- Preventive Maintenance inspections and repairs to include, but not limited to:

- Total checking the existing system

- Cleaning of devices and controls (detector)

- Fixing troubles

- Reprogramming all system as required

- Corrective maintenance as required

- Examine batteries and their connections and test them as specified by supplier to ensure that it is not likely to fail before the next inspection

- Check alarm functions of the panel by operating a detector or call point in each zone

- Check alarm sounders and automatic link to remote centers, as required

- Check all fault indicators and circuits by simulating a fault possible

- Visually inspect whether structural or occupancy changes have affected the requirements for the sitting of call points, detectors and sounders

- Visually inspect to confirm that a clear space of at least 750mm is preserved in all directions below each detector, that detectors are properly sited and that all call points remain unobstructed and conspicuous

- Provide report and submit corrective action plan

- Conduct training to relevant ASEC staff

The equipment installed at new building as follow:

| NO | FLOOR LEVEL | Number of Alarm Detector | Number of Smoke Head Detector | Number of Detector | Number of Alarm Module | Number of Head Detector |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | Rooftop Tower South | 3 |  | 1 |  |  |
| 2 | Rooftop Tower North | 3 |  | 1 |  |  |
| 3 | Level 15 Tower South | 18 |  | 1 | 1 |  |
| 4 | Level 15 Tower North | 19 |  | 1 | 1 |  |
| 5 | Level 14 Tower South | 19 |  | 1 | 1 |  |
| 6 | Level 14 Tower North | 20 |  | 1 | 1 |  |
| 7 | Level 13 Tower South | 19 |  | 1 | 1 |  |
| 8 | Level 13 Tower North | 19 |  | 1 | 1 |  |
| 9 | Level 12 Tower South | 20 |  | 1 | 1 |  |
| 10 | Level 12 Tower North | 20 |  | 1 | 1 |  |
| 11 | Level 11 Tower South | 18 |  | 1 | 1 |  |
| 12 | Level 11 Tower North | 19 |  | 1 | 1 |  |
| 13 | Level 10 Tower South | 19 |  | 1 | 1 |  |
| 14 | Level 10 Tower North | 19 |  | 1 | 1 |  |
| 15 | Level 9 Tower South | 14 |  | 1 | 1 |  |
| 16 | Level 9 Tower North | 21 |  | 1 | 1 |  |
| 17 | Level 8 Tower South | 16 |  | 1 | 1 |  |
| 18 | Level 8 Tower North | 19 |  | 1 | 1 |  |
| 19 | Level 7 Tower South | 16 |  | 1 | 1 |  |
| 20 | Level 7 Tower North | 26 |  | 1 | 1 |  |
| 21 | Level 6 Tower South | 22 |  | 1 | 1 |  |
| 22 | Level 6 Tower North | 17 | 4 | 1 | 1 |  |
| 23 | Level 5 Tower South | 22 |  | 1 | 1 |  |
| 24 | Level 5 Tower North | 15 |  | 1 | 1 |  |
| 25 | Level 4 Tower South | 17 |  | 1 | 1 |  |
| 26 | Level 4 Tower North | 17 |  | 1 | 1 |  |
| 27 | Level 3 Tower South | 24 |  |  | 1 |  |
| 28 | Level 3 Tower North | 19 |  |  | 1 |  |
| 29 | Level 2 Tower South | 32 |  |  | 1 |  |
| 30 | Level 2 Tower North | 25 |  |  | 1 |  |
| 31 | Level 1 Tower South | 21 | 4 |  | 1 |  |
| 32 | Level 1 Tower North | 25 |  |  | 1 |  |
| 33 | Level GF Tower South | 13 |  | 3 | 1 |  |
| 34 | Level GF Tower North | 17 |  | 1 | 1 |  |
| 35 | Basement 1 South | 25 |  |  | 1 |  |
| 36 | Basement 1 North | 19 | 10 |  | 1 | 4 |
| 37 | Basement 2 South | 25 |  |  | 1 |  |
| 38 | Basement 2 North | 26 |  |  | 1 |  |
| 39 | Podium 1 | 3 |  |  |  |  |
| 40 | Utility & Site Plan | 1 |  |  |  |  |

- Requirement for Submission

- Technical

  - Contractor must have minimum 5 years’ experience in managing Siemens fire alarm system (evidence is required)

  - Contractor is highly recommended to attend preliminary site visit to check existing condition to ensure that the proposal submitted will cover all the possible condition. ASEC will not be responsible to cover additional cost due from differing site condition.

  - Contractor must submit Certificate/Letter from Principle of Siemens Indonesia

- Financial

Provide bid amount in IDR (for local bidder)/USD (for international bidder) covering all the requirements as elaborated in the TOR.

- Work Schedule

The work shall be finished in before end of December 2022, and the work schedule in accordance with ASEC procedures, which is:

- Saturday, Sunday, and holiday: 07.00 till 22.00

- The vendor must follow the Covid-19 Preventive Measures and Health Protocol in ASEC, as follows:

- Workers coming into ASEC Premises are required to present a proof of antigen or PCR test with a result “non-reactive / negative” taken within 72 hours and shall register his/her ID card

- Wearing mask is compulsory for all personnel in ASEC Premises. Security guards may deny entry to ASEC building to anyone who does not wear a mask

- Workers are required to have their temperature checked prior to their commencement of works. Personnel with temperature more than 37.2 C will be denied entry into ASEC Premises.

*****