1.0 	Multiple Pole

The requirement is to build a customised multiple pole holder that can hold up to 12 room flags consisting of AMS and ASEAN Flag. The Multiple Pole Holder is to be used at the Lobby of the New Building.

It must be strong enough with sufficient backing to hold 12 flag pole and 12 numbers of flag with size of 1m by 1.5 m and movable using heavy duty wheels with lock.

The requirement is for 1 units of Multi Pole Flag Holder as per technical specifications below.

2.0	Technical Specifications

Quantity	1 Units Multi Flag Holder

Material	All material must be Polished Mirror Stainless Steel 304 with minimum thickness of 1.5 mm (except for castor wheel).

Dimension	Please refer to Diagram 1 – 5 for dimensions.

Main Frame	Square Hollow size minimal 40 cm X 40 cm x 1.2 mm

Flag Holder	12 Round Slots for flag pole with thickness of not less than 1.5 mm and to fit pole size 3.8 mm and length of not less than 45 cm.

Flag Pole	12 units of Round hollow pipe which can be inserted to the flag holder. Size must fit the flag holder accordingly and must not be loose. Length of flag pole must be 240 cm

Finishing	All weld must be neat. All joints must be welded without any gap. All stainless steel parts must be mirror finish.

Wheel	4 to 6 units of Heavy Duty Wheel for each Multiple Pole Frame, Wheels must have lock and rotates 360 o.

LOGO	10 pieces of separated emblem of stainless steel as per attached Diagram. The space between each emblem must be proportionate to the entire Frame.